| Fuel Control Unit |  | P/N: 4138008 / 3244872 | S/N: |
| --- | --- | --- | --- |
| Model: DP-F2 |  | Data: | O/S: BEL |
| Client: |  |  | Pag: 1 de 3 |
| Manual*: 3244872 | Rev. 2 | Date: Nov. 18, 2020 | ATA: 73-20-64 |

Inspection and Check Fuel Control Unit I.A.W CMM*

| Inspection and Check of Drive Body Assembly |  |  |  |  |
| --- | --- | --- | --- | --- |
| N° | Task | Page | Executor | Inspetor |
| 1 | Inspect components of drive body assembly (60, IPL Fig.1) as outlined in paragraph 1, preceding, and specific components as detailed in Table 502, and as follows: | 501 |  |  |
| 2 | A. Inspect drive shaft (320, IPL Fig. i) as follows: NOTE: This check is to be carried out on a standard surface table. (1) Place drive shaft (320) (drive end) in a standard Vee block. (2) Position dial indicator over extreme end of drive shaft(bearing assembly end). | 501 / 502 |  |  |
| 3 | (3) Zero dial indicator. (4) Rotate drive shaft (320) through 360 ° in Vee block and note amount of movement indicated on dial indicator. (5) If reading is more than 0.002" (0.005 cm), drive shaft(320) is unserviceable and must be replaced. | 502 |  |  |
| 4 | B. Inspect new tube (325) outside diameter after assembly to drive shaft (320) as follows: (1) Inspect tube for signs of shredding or scratching after assembly (Ref. ASSEMBLY, paragraph 2.M.). (2) Check that drive shaft assembly (315) freely enters a0.131" (0.333 cm) diameter standard gage. | 502 |  |  |
| 5 | C. Inspect spline of drive shaft assembly (315) as indicated in Table 502 and Figure 501. | 502 |  |  |
| 6 | Inspect components of flow body assembly (400, IPL Fig. 1) as outlined in paragraph 1., preceding, and specific components as detailed in Table 502, and as follows: A. Leak test torsion shaft assembly (995, IPL Fig. 1) as follows (Ref. Fig. 502): (1) Install torsion shaft assembly (995, IPL Fig.1) in torsion shaft assembly leak test fixture (20, Fig. 901). (2) Apply air or nitrogen at 1000 ± 25 psig (6895 ± 172 kPag) to torsion shaft assembly leak test fixture (20). (3) Submerge pressurized torsion shaft assembly into alcohol, naphtha or fuel for one minute. No leakage is permissible. (4) Remove fixture from fluid; release pressure; remove torsion shaft assembly from fixture; dry parts with dry, filtered, low pressure compressed air. | 503 |  |  |
| 7 | B. Leak test bellows assembly (1210, IPL Fig.1) as follows (Ref. Fig. 503): (1) Install bellows assembly (1210, IPL Fig. 1) into bellows leak test fixture (22, Fig. 901). (2) Connect vacuum source as illustrated in Fig. 503. (3) Apply 25 inches (635 mm) of mercury (Hg) vacuum to bellows leak test fixture (22, Fig. 901) for three minutes. (4) After bellows assembly (1210, IPL Fig. i) has been evacuated of air, observe bubble jar of test fixture for air bubbles, indicating leakage. No leakage is permissible. | 504 |  |  |
| 8 | C. Measure bellows assembly (1210) evacuated bellows height as follows (Ref. Fig. 504): (1) Using bellows height check fixture (21, Fig. 901), place master on stand in place of bellows assembly (1210, IPL Fig. 1) and zero dial indicator gage. CAUTION: DO NOT TIGHTEN TO POINT WHERE BELLOWS DISTORTION OR DAMAGE WILL RESULT. (2) Slide stand out from under dial indicator, remove master and install bellows assembly (1210) onto stand, finger tight, ensuring bellows assembly is screwed firmly into stand. (3) Place bellows assembly (1210) and stand under dial indicator and record indicated bellows height. (4) Observe ambient barometric pressure and determine correction factor from Table 501. | 505 |  |  |
| 9 | (5) Add or subtract correction factor to/from dial indicator reading and compare resultant height dimension inscribed on bellows assembly end plate. Resultant height must be within 0.007" (0.018 cm) of bellows assembly inscribed height. If resultant height is not within inscribed height limit, bellows assembly (1210, IPL Fig.1) is unserviceable and must be replaced. Measure replacement bellows height as per this paragraph. D. Measure height of bimetallic disc group (1365) as follows: NOTE: Bimetallic disc group (1365) consists of six separate discs. (1) Place each two discs with concave surfaces facing each other and stack the three sets of two. (2) Height of bimetallic disc group (1365) stack must be0.270" ± 0.020" (0.686 cm ± 0.051 cm), measured at 26.7°C (80°F) temperature. | 507 |  |  |
|  | Spring Test Limits |  |  |  |
| 10 | Testing of all Main Fuel Control Unit springs is not mandatory however, if it is desirable to confirm the serviceability of any spring, spring test limits are listed in Table 503. | 507 |  |  |

Obs:__________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________