|  | BELLOWS CONTROLS SERVIÇOS AERONÁUTICOS. COM 1712-31/ANAC FICHA DE TESTE FCU |
| --- | --- |

| Unidade: FCU |  | P/N: 4138008-3 | S/N: F76626 |
| --- | --- | --- | --- |
| Modelo: DP-F2 |  | Data: 03 / 02 / 2023 | O/S: BEL 0029 / 2023 |
| Cliente: AERO CHAPADÃO |  |  | Pag: 1 de 9 |
| Manual*: 3244872 | Rev. 2 | Data: Nov. 18, 2020 | ATA: 73-20-64 |

TEST SPECIFICATION Nº AETS 71187 - Rev. A

|  | H. All adjustments are to be properly torqued. Test points indicated (*) require special attention and are to be checked following proper torquing. 1.; Metering valve orifice torque; 35; 40 in. Ibs.. 2.; By-pass setting torque; 25; 35 in. Ibs.. 3.; Min. M.V..stop (breakaway) torque; 4; 20 in. Ibs.. 4.; Max. flow M.V. stop torque; 20; 25 in. Ibs.. 5.; Max. gov. eccentric torque; 7; 7.5 in. Ibs.. 6.; Max. throttle, stop torque; 20; 25 in. Ibs.. 7.; Idle speed stop torque; 20; 25 in. Ibs.. 8.; Flow body to drive body; 20; 25 in. Ibs.. 9.; Drive body adapter to drive body; 25; 30 in. Ibs. in 5 in. Ibs. increments. 10.; Py Bleed; 9; 11 in. Ibs.. 11.; Man. Override stop nut; 20; 25 in. Ibs,. I. Speed for Test Points indicated by (#) is to be set to + 2 RPM of specified value. All other points are to be set within + 20 RPM of specified value. J. Total supply is to be set to ± 20 PPH of the specified value or within limits when shown. K. Before calibration, check for external air leakage using M73/9. Slow bubbling is permitted at the joint of drive body and flow body, at the joint of drive body and drive body adapter, at the torsion shaft cover and at the bellows and torsion shaft Iocknuts. No leakage is permitted at any other point. Pc = 100 psia. If leakage is considered excessive, measure Px-Py when Pc = 100 psia, RPM = 0, and T/A = Max. Px to be measured at the bellows ratio lever cover. Px- Py = ± 1.2" Hg. Maximum L. Maintain manual override fully C.C.W, except where otherwise indicated. |
| --- | --- |

| 1.01 | CHECK RELIEF VALVE OPENING (a) Cap outlet port. (b)Increase fuel flow until supply is within limits specified and observe P1-Po differential. (c) If P1-Po differential is out of limits, shim as required. Add shims to increase differential. 1; 5; 2; 3; 4. 0; 200-275; MAX; +105; 85.0. Relief valve opening P1-P0 950 1000 psi |
| --- | --- |
| 1.02 | CHECK PRESSURIZING VALVE OPENING (a) Outlet line connected to control. (b) Increase supply pressure with outlet port open, observe start of nozzle flow. (c) If P1-Po is out of limits, shim as required. Add shims to increase differential. 1; 5; 2; 3; 4. 0; 100; MAX; +105; 85.0. Pressurizing valve opening P1-P0 55 68 psi If valve cover has been removed and reassembled, torque screws to 4.5 - 5.0 in-lb, in 1 in-lb, increments maximum while alternating between the screws. |

|  | METERING VALVE ORIFICE AND BY-PASS HEAD (P1-P2) "HG SETTING (a) Test points 2.01 thru 2.03 are interrelated and should be recorded after all three (3) adjustments have been made. (b) If at T.P. 2.03, P1-P2 is higher than 41 in. Hg, remove by-pass valve cover and replace existing spacer PIN 3240905 with thicker spacer PIN 3240905-1. If P1-P2 is lower than 33 in. Hg., remove and discard existing spacer. (c) Re-torque the by-pass valve retaining screws to 20 in-lb in increments of 5 in-lb and recalibrate T.P.2.01 to 2.03. |
| --- | --- |
| 2.01 | 3; 4-6; 1; 2; 5. 5800; 1350; MAX; +105; 40.0. 203 212 Adjust head and orifice, P1-P2 = 26-42 in HG. |
| 2.02 | 3; 4-6; 1; 2; 5. 2900; 720; 30-32; +105; 40.0. 80 84 Pn = 75-90 psig Set minimum MN stop after removing Py line. Open back pressure valve. |
| 2.03 | 3; 4-6; 1; 2; 5. 5800; 1350; MAX; +105; 70.0. 367 373 Retrim head and re-adjust 2.01 – 2.02 if out of limits P2= 365 415 psig Pc-Px 1.0 in Hg. Max Px-Py 1.2 in Hg. Max As per note (b), check appropriate box. Original spacer Spacer removed Spacer replaced |
| 2.04 | 3; 4-6; 1; 2; 5. 5800; 1350; MAX; +105; 100.0. Set maximum M/V stop 506 520 |
| 2.05 | 3; 4-6; 1; 2; 5. 5800; 1350; MAX; +105; 40.0. -10 +10 |

|  | ENRICHMENT SPRING SETTING |
| --- | --- |
| 3.01 | 3-7; 4-6; 1; 2; 5. 5800; 1350; MAX; -50; 55,0. X X Record Nozzle Flow |
| 3.02 | 3-7; 4-6; 1; 2; 5. 5100; 1170; MAX; -50; 55,0. -13 -17 Adjust enrichment spring to obtain 13 to 17 PPH less than 3.01. Increasing spring tension increases difference. To verify setting, approach enrichment speed from 4600 RPM. (Re-check T.P. 3.01 & 3.02 after Adjustment) |
| 4.01 | 3; 4-6; 1; 2; 5. 4000; 1000; MAX; -50; 40,0. 172 183 Pn= 100-110 psig. Pc-Px 8.5 11.5"HG Px-Py 1.2" HG Max. |

|  | MAX. GOVERNOR SPRING SETTING (a) Adjust eccentric shaft to meet flow limits. (b) If unable to set with eccentric, maximum T.A. may be varied within T.A. limits. (c) If unable to set with eccentric or maximum T.A. maintain eccentric shaft scribe line and determine amount of RPM off. (d) Split control and adjust gov. spring as required. (CW adj. of governor spring delays break, approximately 1 flat on adj. nut per 50 RPM). (e) If gov. slope is not in limits, select Py bleed. PIN 2525107 Py #58 #66 (higher number bleed to increase difference in flow). |
| --- | --- |
| 5.01#* | 5; 4-7; 1; 2; 3-6. 6350; 1440; +59; 85.0. 325 355 Set governor break T.A, at 89° - 91° Px-Py "HG |
| 5.02#* | 5; 4-7; 1; 2; 3-6. 6500; 1530; +59; 85.0. X X 105 125 Diff. from T.P. 3.01 (Gov. slope) |
| 5.03 | 5; 4-7; 1; 2; 3-6. 1440; "; +55; 85.0. X X Hysteresis within -50 0 RPM of T.P. 5.01 Px-Py "HG |

|  | IDLE SPEED STOP SETTING AND PICK-UP ANGLE (a) At test point 6.02 Increase T.A. until 4 PPH max. fuel flow from 6.01 is observed. (T.A. where flow starts to increase is pick-up angle). If throttle cam pick-up angle is out of limits, replace governor spring. (1) • Use a higher numbered spring if pick-up angle exceeds 56 ° diff. from T.P. 6.01. (2) Use a lower numbered spring if pick-up angle is less than 52 ° diff. from T.P. 6.01. (3) Adjust spring tension to same load as set on spring removed. (4) Re-set gov. break at test point 5.01 and rerun T.P.'s 5.02 and 5.03 inclusive. (5) Re-set idle stop screw at test point 6.01. |
| --- | --- |
| 6.01 | 4; 1-7; 5; 2; 3-6. 3200; 800; 30-32; +59; 30.0. 104 112 Set idle stop screw (CW adj. to increase flow). Decrease and re-set speed after adjustment throttle pick-up angle. |

| 6.02 | 4; 1-7; 5; 2; 3-6. 3200; 800; +59; 30.0. Pn = 95 - 105 Throttle pick-up angle difference 52° 56° of T.P. 6.01 Spring Stripe |
| --- | --- |

|  | LOW END OF ACCELERATION FLOW AND THROTTLE MOVEMENT EFFECT (a) After recording nozzle flow at test point 7.02 move throttle to 30° T.A. (b) Observe nozzle flow change and record. |
| --- | --- |
| 7.01 | 3; 5; 4; 2; 1. 815; 200; 40; +59; Local. 60 X Pn= 75 psig |
| 7.02 | 3; 5; 4; 2; 1. 2100; 670; 40; +59; 25.0. 113 124 Pn= 95-105 psig Move T.A. to 30° 5 PPH maximum decrease |

|  | MAX. IDLE RESET STOP ADJ |
| --- | --- |
| 8.01 | 1; 4-6; 5; 2; 3-7. 5000; 1170; -; +59; 55.0. 188 197 Set Max. idle reset stop. Record T.A. |

|  | MANUAL OVERRIDE At test point 9.01 set max. manual override flow. Rotate override lever C.W. and maintain in contact with max. flow adjusting screw. Rotate adjusting screw C.C.W. to increase flow. |
| --- | --- |
| 9.01 | 1; 5; 2; 3; 4. 5000; 1170; 32; +105; AMB.. 405 415 |
| 9.02 | Record manual override total angular travel 94° 104° |

|  |  | CUT-OFF MODULATION AND CUT-OFF LEAKAGE A. At test point 10.01, set test condition with 20 ° T.A. (a) Move throttle to 30 ° T.A. and observe nozzle flow change. If nozzle flow change is excessive - (1) Reset throttle to 20 ° T.A. (2) Back off override adj. nut on cut-off link. (3) Adjust cut-off link longer to meet max. flow change limit. |
| --- | --- | --- |
| 10.01 |  | 3; 4-6; 1; 2; 5. 2000; 500; 20; +105; A.R.. 160 170 Nozzle flow change 4PPH max. |
|  | B) At test point 10.02 check cut-off leakage. (a) Remove nozzle outlet from control. (b) Set total supply with throttle in cut-off position. (Highest T.A. position where no leakage is observed at outlet fitting). (c) Increase T.A. to permit flow out the outlet fitting. (d) Decrease T.A. as required until no leakage is observed at outlet fitting (Observe T.A.). (e) Same Pc as established for T.P. 10.01. |  |
| 10.02 | 3; 4-6; 1; 2; 5. 2000; 500; 8-10; +105; -. P1-P2 0 0.5 cc/min. Cut-off leakage P1 pressure 250 psig max. |
|  | C) At test point 10.03 (-5 ° T.A.). (a) Observe P1 pressure. (b) Upper lever (idle reset lever) stop to just contact lever. (c) At 0 ° T.A. lower C.O. lever to be within ± 5 ° of horizontal. (d) Pc same at T.P. 10.01. |  |
| 10.03 | 3; 4-6; 1; 2; 5. 2000; 500; -5; +105; AR. P1 pressure equal to or less than T.P. 10.02 P1 psig |

| 10.04 | 3; 4-6; 1; 2; 5. Min-Max-Min; Local. Operate throttle. Lever over full range. Throttle torque 15 in-lb max. |
| --- | --- |

|  | EXTERNAL LEAKAGE CHECK A. Cap Po at drive flange by-pass tap. B. Remove Po line to control and install Po plug. C. Cap outlet port. D. Set P1 pressure to 1000 PSIG. E. Observe leakage for 2-3 minutes. F. No leakage permitted. |
| --- | --- |
| 11.01 | OK REJECT |
| 11.02 | Record shipping weight X 5.3 lbs.max |
|  | Checado por :____________________ Carimbo/Rubrica Inspecionado por:_______________________ Carimbo/Rubrica Data de encerramento do teste:______________________ |